CIT620 COMPUTER SUPPORT AND MAINTENANCE

NAMES:KATLEHO QHOBELA

STUDENT NUMBER:20250016

WEEK3 ASSIGNMENT

1. Hardware Component Analysis for IT Scenario

Scenario: Building a cost-effective, reliable PC for IT support technicians who run VMs, VS Code, Docker, and remote desktop tools. Must last 4+ years.

Component	Chosen Spec	Type/Characteristics	Why for IT/Programming	Maintenance

Motherboard	ASUS B660M-A D4 mATX	Chipset: Intel B660. Socket: LGA1700. Slots: 2x M.2, 4x SATA, PCIe 4.0	mATX fits standard IT dept cases. M.2 supports fast NVMe for VM boot. 4 RAM slots allow 64GB upgrades.	Dust with compressed air 2x/year. Update BIOS for CPU compatibility. Check for swollen capacitors.

CPU	Intel i5-12400	6 Cores / 12 Threads. Base 2.5GHz, Boost 4.4GHz. TDP 65W. No iGPU	12 threads handle Docker + VMs + IDE. 65W TDP = low heat in office PCs. No iGPU saves cost since IT uses discrete GPU or remote.	Reapply thermal paste every 2 years. Monitor temps with HWMonitor <80°C. Clean stock cooler.

RAM	2x16GB DDR4-3200 CL16	Type: DDR4 DIMM. Dual-channel. Latency: CL16	32GB total lets techs run 2-3 VMs + Chrome + VS Code. Dual-channel doubles bandwidth vs single stick. DDR4 = cost-effective.	Reseat if POST fails. Run MemTest86 yearly. Keep in matched pairs.

Storage	1TB NVMe SSD + 2TB HDD	NVMe: PCIe 3.0, 3500MB/s read. HDD: 7200RPM SATA	NVMe for OS + code + VMs = 10s boot, fast compile. HDD for ISO images, logs, backups. Tiered storage saves budget.	Check SSD health with CrystalDiskInfo. Defrag HDD only. Keep 20% free on NVMe.

Tools Used: CPU-Z to verify RAM running dual-channel, CrystalDiskInfo for drive health, Canva for diagram.

2. Diagram Description for Canva

_Title: Programming PC Hardware Layout_

Layout to recreate in Canva:

1. Center: Rectangle labeled “ASUS B660M-A Motherboard”

2. Top Center: “Intel i5-12400 CPU” inside socket with arrow pointing down to motherboard. Label “LGA1700”. Add cooler icon.

3. Right Side: 2 rectangles “16GB DDR4 RAM” in Slot A2 + B2. Arrow to motherboard “Dual Channel”. Label “3200MHz”.

4. Bottom Right: “1TB NVMe SSD” in M.2 slot. Arrow to motherboard “PCIe 3.0 x4”.

5. Bottom Left: “2TB HDD” with SATA cable arrow to “SATA Port” on motherboard. Add power cable arrow to “PSU”.

6. Callouts: Add text boxes: “CPU: Runs code, VMs”, “RAM: Active programs”, “NVMe: Fast OS boot”, “HDD: Bulk storage”.

7. Legend: Use icons: CPU chip, RAM stick, SSD, HDD. Color code: Blue = fast, Green = bulk.

3. 200-Word Explanation

Scenario: IT support staff need reliable PCs to troubleshoot, code scripts, and run test VMs. This build balances cost, speed, and longevity for a university IT department.

Concept: The motherboard is the backbone, connecting all parts via the B660 chipset. It determines upgrade paths like 4 RAM slots and M.2 support. The CPU i5-12400 uses 6 performance cores to compile code and run Docker containers simultaneously. Its 65W TDP ensures quiet office operation. RAM type DDR4 in dual-channel gives 51.2GB/s bandwidth, critical when multiple VMs access memory. 32GB prevents swap file slowdowns during debugging. For storage, an NVMe SSD uses PCIe lanes for 3500MB/s speeds, making Windows + WSL boot in seconds. The SATA HDD provides cheap bulk storage for ISO files and ticket logs.

Maintenance: IT must monitor CPU temps under 80°C to prevent throttling, test RAM yearly with MemTest86 to avoid blue screens, and check SSD wear levels to plan replacements before failure. Tools like CPU-Z verify hardware runs at spec.

Significance in IT: Understanding components lets IT support diagnose slow PCs, plan lifecycle refreshes, and avoid overpaying. A tech who knows dual-channel RAM can fix a “slow” PC by adding a matching stick vs ordering new. This knowledge cuts downtime and department costs. Hardware analysis is core to system building, asset management, and Level 2 support.