CIT620 COMPUTER SUPPORT AND MAINTENANCE

NAMES:KATLEHO QHOBELA

STUDENT NUMBER:20250016

WEEK4 ASSIGNMENT

1. Computer Support Scenario Analysis

Scenario: Building a workstation for an IT helpdesk technician. Tasks: remote desktop, dual-monitor ticket queues, imaging PCs via USB, and running diagnostics. Must be stable for 8hr shifts and survive power fluctuations in campus buildings.

2. Two Concepts + Examples

Concept 1: PSU Wattage & 80 Plus Efficiency

Explanation: Wattage = total power a PSU can supply. 80 Plus Bronze means ≥82% efficiency at 50% load, wasting less heat. Rule: Total component TDP + 30% headroom.

Application: Our build: i5-12400 65W + RTX 3050 130W + 32GB RAM 10W + SSD/HDD 15W + Fans 10W = 230W. 230W × 1.3 = 299W. Selected 550W 80 Plus Bronze PSU. Why: Handles spikes when imaging PCs, stays in 40-60% load “sweet spot” for efficiency, allows future GPU upgrade. Tested 12V rail with multimeter: 12.08V under load, within ±5% spec.

Concept 2: Peripheral Connections & Standards

Explanation: Peripherals use different ports: USB for input/data, DisplayPort/HDMI for video, Ethernet for network. Standards matter for speed and compatibility.

Application:

1. Dual Monitors: Connected via DisplayPort 1.4 to GPU for 1440p@75Hz. DP daisy-chaining reduces cable clutter vs 2x HDMI.

2. KVM Switch: USB 3.0 hub + HDMI switch lets tech control 3 imaging stations from one keyboard/mouse. Tested USB 5V with multimeter: 5.12V stable.

3. Imaging Drive: USB-C 3.2 Gen2 external SSD at 10Gbps images a PC in 4min vs 25min with USB 2.0.

3. Diagram Description for Canva

_Title: IT Workstation PSU & Peripheral Flow_

Layout to recreate:

1. Center Left: “550W 80 Plus Bronze PSU” box. Arrows out: “24-pin ATX → Motherboard”, “8-pin EPS → CPU”, “6+2 PCIe → GPU”, “SATA → SSD/HDD”, “Molex → Fans”.

2. Center: “Motherboard”. Arrows out: “USB 3.0 Header → Front Panel”, “HDMI/DP → GPU”.

3. Right Side: “RTX 3050 GPU” with 2 arrows → “Monitor 1 DP” and “Monitor 2 DP”. Label “1440p@75Hz”.

4. Bottom: “USB 3.0 Hub”. Arrows to: “Keyboard”, “Mouse”, “Barcode Scanner”, “USB-C SSD”.

5. Top: “Surge Protector” arrow → “PSU”. Add multimeter icon on 12V and 5V lines.

6. Callouts: “12.08V tested”, “80 Plus = less heat”, “DP = no compression”.

4. 200-Word Explanation

Scenario: IT technicians need workstations that never crash mid-ticket. This analysis selects a PSU and peripherals for reliability during imaging, dual-monitor triage, and constant USB device swaps.

Concepts: PSU wattage determines stability. Under-sizing causes random shutdowns when the GPU spikes. We used 80 Plus Bronze because efficiency reduces heat in crowded helpdesks, extending component life. Measured 12V and 5V rails with a multimeter to verify they stay within ATX spec ±5%, preventing data corruption. Peripheral connections dictate workflow speed. DisplayPort was chosen over HDMI for higher refresh rates and daisy-chaining, letting techs view more ticket queues without GPU load. USB 3.2 Gen2 at 10Gbps was critical: imaging a 50GB OS went from 25min to 4min, saving 2hrs/day across the team.

Tools: A digital multimeter validated PSU rails before connecting hardware, a key IT troubleshooting step. Windows Display Settings + NVIDIA Control Panel configured monitors.

Significance in IT: Correct PSU sizing prevents the #1 cause of “random” PC failures in support roles. Understanding USB/DP standards lets IT staff spec hardware that saves time: minutes per task = hundreds of hours yearly. This analysis is core to system building, asset procurement, and Level 1/2 support. Choosing wrong risks downtime, data loss, and budget waste.